| Risk Assessment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Risk Assessment for the activity of | Hand-held Spotwelding |  | Date | 09/03/2026 |
| Group Name | RESUS- Renewability and sustainability society | Assessor | Yousra Igouzal |  |
| Supervisor | Aaron campbell | Signed off |  |  |
| Description of event/activity | Wireless Hand held spotwelding Li-po batteries together in a university lab (activity will occur in B23/2045 Design Studio, reoccurring weekly activity) |  |  |  |

| PART A |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (1) Risk identification |  |  | (2) Risk assessment |  |  |  | (3) Risk management |  |  |  |
| Hazard | Potential Consequences | Who might be harmed (user; those nearby; those in the vicinity; members of the public) | Inherent |  |  |  | Residual |  |  | Further controls (use the risk hierarchy) |
|  |  |  | Likelihood | Impact | Score | Control measures (use the risk hierarchy) | Likelihood | Impact | Score |  |
| Contact with hot spot welder tip, nickel or batteries. | Burns to hands/fingers/other body parts | User | 4 | 2 | 8 | Tweezers should be used to hold nickel strips in place. Spot welder shouldn’t be used in the same place more than a few times or that material will heat up too much. User should avoid touching the tip of the spot welder, nickel, batteries or any metal the spot welder has touched while it is still hot. It should be assumed any metal is hot if unknown. | 2 | 2 | 4 | Run any burns under water. |
| Battery damage | Fire, explosion, toxic inhalation. | User and those nearby | 1 | 4 | 4 | Battery should not be heated up. Eye protection should be worn. Batteries must be measured before use. Known damaged batteries should be informed by staff so they can be disposed of appropriately. Batteries must be stored charged and stored in fireproof bags. | 1 | 2 | 2 |  |
| Combustible items coming in to contact with spot welder | Fire, burns, inhalation of smoke/fumes. | User | 1 | 2 | 2 | Always keep work area tidy. Ensure that combustible/flammable items (e.g. paper, clothing, flammable substances) are stored well away from the hot soldering iron/work area. Check the work area is in a safe state when work has been completed. Switch soldering iron off after use. | 1 | 1 | 1 |  |
| Use of hand tools e.g. cutters, pliers, blades, screwdrivers etc. | Cuts, nips and pinches to hands and fingers. Injury to eyes from cutting component legs incorrectly | user | 1 | 2 | 2 | Wear protective glasses When trimming component legs, point towards the floor or into a waste container/bin. Visually inspect hand tools prior to use. If tools are damaged, do not use and contact technical staff for replacement. | 1 | 1 | 1 |  |
| Fumes | Eye and nose irritation, damage to the air passages and/or respiratory irritation Existing health problems, i.e. asthma, COPD may become worse during soldering. | User | 1 | 2 | 2 | Use fume extraction and ensure adequate ventilation when soldering. If soldering activites are likely to take longer than 15 continuous minutes, speak to supervisor /technical staff to discuss further controls. Check that the bench top fume extractor unit is operating correctly before soldering. Check that the filter is not clogged. Filter colour should be black. If grey in colour, contact local technical staff to fit replacement Filter. Lead free solder supplied and available from the EEE Stores. Tin/lead solder must not be used. | 1 | 1 | 1 | If any minor symptoms occur due to fumes, it should be managed by the lab’s first aid kit and staff. |
| Electrical shock | Risk of electric shock | User | 1 | 3 | 3 | No water should be near when using electrical equipment and whoever is using any, must make sure their hands are dry. Wear protecting equipment when dealing with open wires. | 1 | 2 | 2 |  |
| Physical | Upper limb discomfort, tiredness, loss of concentration may occur when soldering for long periods of time e.g. more than 15 minutes | User | 1 | 2 | 2 | Adjustable height seating provided. Ensure that a break or change of activity is made for at least 5 minutes every hour. Organise the work bench/area in a way that provides easy access to all items needed for the construction of the equipment. | 1 | 1 | 1 |  |
| Slips, Trips, Falls | Injury, burns to user, burns to others, damage to property. | User and those nearby | 2 | 2 | 4 | Ensure the device is only switched on when the user is seated. Only walk with the device when it is cool and packed away. | 1 | 1 | 1 |  |

| # PART B – Action Plan |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| # Risk Assessment Action Plan |  |  |  |  |  |  |  |
| # Part no. | # Action to be taken, incl. Cost | # By whom | # Target date |  | # Review date | # Outcome at review date |  |
|  | # Will ensure that all users have read both the risk assessment and method statement and are aware to contact B23 technicians in event of safety risk | # Sophie Ridgewell | # 12/3/26 |  | # 17/9/26 |  |  |
|  | # Investigating chosen workshop to ensure suitable safety measures for the operating voltage level of the device are in place | # Sophie Ridgewell | # 12/3/26 |  | # 17/9/26 |  |  |
|  | # Ensure all are informed of how to use the device safely | # Sophie Ridgewell | # 19/3/26 |  | # 17/9/26 |  |  |
| # Responsible manager’s signature: ACampbell |  |  |  |  | # Responsible manager’s signature: |  |  |
| # Print name: Aaron Campbell |  |  |  | # Date: 9/3/26 | # Print name: Sophie Ridgewell |  | # Date: 6/3/26 |

Assessment Guidance

| - Eliminate | Remove the hazard wherever possible which negates the need for further controls | If this is not possible then explain why |  |
| --- | --- | --- | --- |
| - Substitute | Replace the hazard with one less hazardous | If not possible then explain why |  |
| - Physical controls | Examples: enclosure, fume cupboard, glove box | Likely to still require admin controls as well |  |
| - Admin controls | Examples: training, supervision, signage |  |  |
| - Personal protection | Examples: respirators, safety specs, gloves | Last resort as it only protects the individual |  |

| LIKELIHOOD | 5 | 5 | 10 | 15 | 20 | 25 |
| --- | --- | --- | --- | --- | --- | --- |
|  | 4 | 4 | 8 | 12 | 16 | 20 |
|  | 3 | 3 | 6 | 9 | 12 | 15 |
|  | 2 | 2 | 4 | 6 | 8 | 10 |
|  | 1 | 1 | 2 | 3 | 4 | 5 |
|  |  | 1 | 2 | 3 | 4 | 5 |
|  |  | IMPACT |  |  |  |  |

| Impact |  | Health & Safety |
| --- | --- | --- |
| 1 | Trivial - insignificant | Very minor injuries e.g. slight bruising |
| 2 | Minor | Injuries or illness e.g. small cut or abrasion which require basic first aid treatment even in self-administered. |
| 3 | Moderate | Injuries or illness e.g. strain or sprain requiring first aid or medical support. |
| 4 | Major | Injuries or illness e.g. broken bone requiring medical support >24 hours and time off work >4 weeks. |
| 5 | Severe – extremely significant | Fatality or multiple serious injuries or illness requiring hospital admission or significant time off work. |

Risk process

- Identify the impact and likelihood using the tables above.

- Identify the risk rating by multiplying the Impact by the likelihood using the coloured matrix.

- If the risk is amber or red – identify control measures to reduce the risk to as low as is reasonably practicable.

- If the residual risk is green, additional controls are not necessary.

- If the residual risk is amber the activity can continue but you must identify and implement further controls to reduce the risk to as low as reasonably practicable.

- If the residual risk is red do not continue with the activity until additional controls have been implemented and the risk is reduced.

- Control measures should follow the risk hierarchy, where appropriate as per the pyramid above.

- The cost of implementing control measures can be taken into account but should be proportional to the risk i.e. a control to reduce low risk may not need to be carried out if the cost is high but a control to manage high risk means that even at high cost the control would be necessary.

| Likelihood |  |
| --- | --- |
| 1 | Rare e.g. 1 in 100,000 chance or higher |
| 2 | Unlikely e.g. 1 in 10,000 chance or higher |
| 3 | Possible e.g. 1 in 1,000 chance or higher |
| 4 | Likely e.g. 1 in 100 chance or higher |
| 5 | Very Likely e.g. 1 in 10 chance or higher |